Cover 1

Coin-operated Massage Chair

User Manual

Cover 2

Thank you for your purchase of this Coin-operated Massage Chair. Before use, please read this USER MANUAL carefully, and pay special attention to the safety precaution to ensure proper operation and use.

Please keep this USER MANUAL properly for further reference.

CONTENTS----------------------------------------------------------------------

SAFETY PRECAUTIONS---------------------------------------------------------------1

FUNCTION AND FEATURES---------------------------------------------------------------3

NAME AND FUNCTION OF EACH PART-----------------------------------------------4

PREPARATION BEFORE USE----------------------------------------------------------9

USE METHOD-------------------------------------------------------------------------10

INSTALLATION METHOD--------------------------------------------------------------11

TROUBLE SHOOTING------------------------------------------------------------------12

PRODUCT STORAGE AND MAINTENANCE-------------------------------------------13

TECHNICAL SPECIFICATIONS---------------------------------------------------------13

1st page:

SAFETY PRECAUTIONS

Safety precautions

Before use, please read this “SAFETY PRECAUTIONS” carefully to ensure proper operation.

The following “SAFETY PRECAUTIONS” is of great help to your correct use of this massage chair.

To clearly indicate the degrees of danger and damage, the classes of accidents caused by incorrect use are classified into “WARNING” and” CAUTION”, so everyone must abide by it.

WARNING   Indicating user may be seriously injured once the massage chair is improperly used.

CAUTION     Indicating the user may be injured or the massage chair may be

damaged once the product is improperly used.

The symbol “ “ indicates the behaviors are prohibited.

The detailed contents are described in following charts. (For example: left symbol indicates No Disassembly.)

The symbol “ “ indicates the behaviors are compulsory and must be abided by.

The detailed contents are described in following charts. (For example: left symbol indicates PULL OUT POWER PLUG.)

WARNING

COMPULSORY

Following people must consult their doctors before use:

1. People who are using electronic medical instrument imbedded in the body, such as cardiac pacemaker etc.

2. Patients are being treated by doctors, especially those who feel uncomfortable.

3. Malignant tumor patients, heart disease patients, and acute patients.

4. Pregnant or menstruating women.

5. Patients with osteoporosis or fracture spine.

6. Patients with disturbance of blood circulation caused by diabetes or some other disease.

7. People with wounded skins or people who need quiet rest.

8. People whose body temperature is over 38 degrees (Febrile Phase).

PROHIBITED

Minors or people with intellectual disabilities are prohibited to use this massage chair without supervision.

Do not let children play on massage chair.

The massage chair should be only used by one person at one time.

Stop using the massage chair if power cord or power plug is damaged, and do not dismantle them for maintenance by yourself.

Do not damage, process, excessively bend, pull hard, twist or knot power cord.

Do not use massage chair on electric blanket or heating device, to avoid causing fire disaster.

No Disassembly

Anyone else, except maintenance personnel specified by our company, should not disassemble, maintain, or remodel this massage chair. If any problem, please contact franchiser immediately.

2nd page:

SAFETY PRECAUTIONS

CAUTION

COMPULSORY

Please do not continuously use the massage chair over 20 minutes, and it is limited to massage a particular part of body in 5 minutes.

Please put the massage chair on flat ground for use.

When lowering legrest or backrest, please do not exert excessive force on it, in addition, please make sure there are no children, animals or other objects under legrest and backrest.

When pulling out power plug, please hold the insulation plug, do not pull power cord.

Please do not move the massage chair on easily damaged surfaces, such as wooden floor and carpet.

Please do not use the power supply that is beyond the specified electric voltage, to avoid causing fire disaster or damaging massage chair.

Before use, please uncover the back cushion to check whether backrest or fabric is damaged; if damaged, please stop using this massage chair immediately, and contact franchiser for repair immediately.

Please do not operate the controller with wet hands, and do not put heavy objects on controller.

A person whose body weight is over 120 kilograms should not this product.

Please do not pull hard fabric, or stab it with knife or sharp objects.

Please do not use other therapeutic equipments together with this massage chair.

During use, if you feel uncomfortable, please stop use immediately and consult your doctor.

Do not use the massage chair for medical treatment.

Do not use the massage chair in damp environment to avoid electric leakage or electric shock.

Please keep the massage chair away from stove, and do not exposure it in direct sunlight.

The massage chair is equipped with heating function, so people who are insensitive to heat must use it carefully.

PROHIBITED

After use, please cut off power.

Do not use the massage chair immediately after meals.

Do not fall into sleep on massage chair, when the massage chair is working.

Do not use the massage chair when you are drunk or feel bad.

Before use, please be sure to take off watch, bracelet and some other accessories, and be sure to have taken out mobile phone, keys, knife, and some other valuables from your pocket.

Do not use gasoline or other corrosive cleaning agent to wipe the massage chair, and do not spray pesticide on it.

Do not sit on backrest, legrest, or armrest to avoid accidental injury or machine fault.

During use, please do not smoke to avoid causing fire disaster.

PULL OUT POWER PLUG

Before maintenance, please pull out power plug. In addition, please do not insert or unplug the power plug with wet hands, to avoid electric shock.

When the massage chair is not in use, please pull out the power plug from socket.

When power is off, please cut off power supply immediately and pull out the power plug from socket.

3rd page:

FUNCTION AND FEATURES

- The back moving up and down unit can fit the body curve automatically; the ergonomically design offers the best massage effect; a coin acceptor is installed in the armrest.

2. Delicate massage features provide for humanistic and overall enjoyable massage effect. The back massage unit can move up and down between neck, shoulder, back and waist performing “knead”, “pummel”, “shiatsu” and “wavelet” massages; meanwhile, the massage speed and width are adjustable.

KNEAD   PUMMEL    WAVELET   SHIATSU

- Airbags at lower body perform “wavy” massage; while airbags at the thigh, buttocks and legs areas will be inflated and deflated alternatively offering repeated air pressure massage.

Airbags for massaging the thigh and buttocks

4. Airbags installed at up and down sides of left and right legrest provide for alternate and circulatory massage with the airbags installed at the middle of legrest.

Airbags for massaging the leg

4th page:

NAME AND FUNCTION OF EACH PART

External structure

Pillow

Armrest

Seat cushion

Legrest

Backrest

Fuse holder

Power socket

Backrest locking lever

Caster

Internal structure

Backrest six-roller massage unit

Airbag installed at seat cushion

Airbag installed at legrest

LCD

Coin acceptor

5th page:

NAME AND FUNCTION OF EACH PART

Coin acceptor

| No. | Name | Function | Note |
| --- | --- | --- | --- |
| 1 | Selector switch | User can set two countries or regions’ coins: A and B. | No need to set |
| 2 | 4 pin socket |  |  |
| 3 | Selector switch | NO (Normal open), NC (Normal closed) | Select “NO (Normal open)” |
| 4 | Selector switch | Output signal (fast) 20ms (medium) 50ms (slow) 100ms | Select “(slow) 100ms” |
| 5 | Not in use |  |  |
| 6 | 2 pin socket | Electromagnetic valve DC 12V |  |
| 7 | LED light | Coordinate with the option light on indication |  |
| 8 | Inching button K1 | Reset coin sampling |  |
| 9 | Nixie tube | Indicate state function |  |
| 10 | Inching button K2 | Status display valve increasing? |  |
| 11 | Inching button K3 | Status display valve increasing? |  |
| 12 | Coin slot |  |  |
| 13 | Coin-blocking exclude button |  |  |
| 14 | Slot of coin rejector |  |  |
| 15 | Electromagnetic valve |  |  |
| 16 | Unit for anti-hooking or anti-stealing |  |  |

All the parameters of coin acceptor have been preset when delivery from factory, so user can use it directly. If necessary, user also can reset the parameters, referring to “Parameter setting and changing of coin acceptor”.

Parameter setting and changing of coin acceptor

Button function description:

K1: Confirm and enter into next setting

K2: Value increasing

K3: Value decreasing

K2+K3: Enter into parameter setting

6th page:

NAME AND FUNCTION OF EACH PART

Operation procedure

- Press K2 and K3 at the same time, about 3 seconds later, the LED will display “A”, then user can enter into parameter setting.

- Press K1, the LED will display “E”, then user can set the value of currency; Press k2 or k3 to increase or decrease the value; the set range is 1-5; press K1 to finish the setting and enter into next setting.

- After finishing up Step 2, the LED will display “H”, then user can go into sampling quantity setting of single currency value; the set range is -20; it is advised to set the quantity over 10.

- After finishing up Step 3, the LED will display “P”, then user can go into currency value signal output setting; the set range is -50.

- After finishing up Step 4, the LED will display “F”, then user can go into accuracy setting; the set range is -20; it is advised to set as 5-8.

- After finishing up Step 5, the first kind currency value setting is finished; if the set value of Step 2 is over 1, it needs to repeat Step3, Step4 and Step 5 to go into next currency value setting until all the currency valve setting is finished; When LED displays “A”, user can go into Step 7.

- Press K1, the LED will display “E”, which means setting is finished, then turn off power; turn on power again to go into currency value learning memory setting.

- Press K1, the LED will display “A”; Press K1 again, the LED will display “A1”.

- Press K1, the left LED will display “A”; Press K1 again, the LED will display “A1”. When the first LED light is lightened, user can go into the first currency value learning memory setting.

10. Every one coin inserting, the LED will display the accumulative total. When learning quantity is equal to the quantity of sampling coin of H setting, the first currency value learning is finished, and all the LEDS are flashing; if the corresponding setting value of E is over 1, the LED will subsequently display “AX” (x=2-5), meanwhile the corresponding X LED lights are lightened, now user can go into No. X currency value learning memory setting, and so on, until all the currency learning memory is finished. When LED displays “0”, the coin acceptor goes into normal working order.

Controller

Button description

---- Enter and set password

---- Previous menu / increase

---- Next menu / decrease

---- Confirm / enter into sub-menu

---- Cancel / back to previous menu

Power-on password

Under standby mode, after connecting the controller to the Coin-operated Massage Chair, the massage chair goes into setting state showing “----“ to remind user to enter the password which consisted by four digital, then press “ “ to confirm the password (Password includes supervisor's password and user's password; default supervisor's password: ABCD; user's password: AAAA ). After successfully verifying the password, the massage chair will automatically select the user level according to the password; while if the system fails to verify the password continuously for three times, the massage chair will be locked and all the buttons will be invalid until the massage chair is restarted. Once the plug of controller is pulled out, the massage chair will exit the set state.

7th page:

NAME AND FUNCTION OF EACH PART

Menu description

Main Menu

Accumulative time display (S, U)

Accumulated amount display (S, U)

Display starting coin quantity (S, U)

Display starting time, unit: minute (S, U)

Display backrest reclining time, unit: second (S, U)

Display supervisor user (S)

Display operator user (U, S)

Sub Menu

Keep pressing “ “ button for 6 seconds to clear the accumulative time (S)

Keep pressing “ “ button for 6 seconds to clear the accumulated amount (S)

Set the starting coin quantity (S, U)

Set the starting time (S, U)

Set backrest reclining time (S, U)

Set supervisor's password (S)

Set user's password (S, U)

Note: S: supervisor   U: user

Accumulative time: If the accumulative time is less than 100 hours, the display format is: HH: MM (HH stands for hour, MM stands for minute). If the accumulative time is more than or equal to 100 hours but less than 10 000 hours, it displays 3-4 digits integral hours (minutes are omitted).

Accumulated amount: If the accumulated amount is less than 10 000 currency value, it displays1- 4 digits integer.

Starting coin quantity: the setting range is 1-10, corresponding1-10 coins.

Starting time: the setting range is 1-15, corresponding 1-15 minutes.

Backrest reclining time: the setting range is 0-30, corresponding 0-30 seconds reclining time.

Password setting: when screen shows “SUP_” (supervisor) or “USE_” (user), press “ “ button to enter password setting, the screen will show ”_ _ _ _” to remind user to enter the four-digit password, press “ “ button to save it, the screen will show ”_ _ _ _” again to remind user to reenter the password,  press “ “ button to confirm. Save the modified password and return to previous menu.

Accumulative time clearing: Under displaying accumulative time state, keep pressing “ “ button, the display screen will show “CLEA” 3 seconds later, and the accumulative time will be cleared 6 seconds later. When the clearance is successful, the screen will show “0”.

Accumulated amount clearing: Under displaying accumulated amount state, keep pressing “ “ button, the display screen will show “CLEA” 3 seconds later, and the accumulated amount will be cleared 6 seconds later. When the clearance is successful, the screen will show “0”.

8th page:

NAME AND FUNCTION OF EACH PART

LCD

Display area of working time, remaining time and parameter setting

After the power of massage chair is on, it shows ”rESt”.

After massage chair resets to the standby state, it shows “0”

When the inserted coin's value does not reach to the set value, it shows “---X” (X: 0-9 optional number).

When the inserted coin's value reaches to the set value, it shows remaining time “XX: Xx” (X: 0-9 optional number).

When massage chair is out of order, it shows the malfunction code (ErrX) (X: 0-3 optional number).

Under standby mode, user can enquiry, set or change the parameter (referring to “controller section” for details); when the setup parameter “_Xx” is flashing, you can press ““button or ““button to change the parameter.

It shows “TIME” during massage chair running.

When enquiring the total time, it shows “TIME” and “TOTAL”.

Display area          When enquiring the total coin quantity, it shows “TOTAL”

and “MONEY”.

for running and       When setting the starting coin quantity, it shows “SET” and “MONEY”

status setting         When setting the starting time, it shows “SET” and “TIME”.

When setting backrest reclining time, it shows “SET” and “TIME”.

MODE ------ Massage mode display area: KNEAD   PUMMEL   SHIATSU    WAVELET

WIDTH

narrow   medium   wide

Under “PUMMEL” and “SHIATSU” massage modes, it shows the arm width of current massage unit: narrow    medium    wide

SPEED   Under “PUMMEL”, “KNEAD” and “WAVELET” massage modes, it shows the speed of current massage unit: slow    medium    fast

Airbag position display

Legrest ascending/descending indication

Backrest ascending/descending indication

9th page:

PREPARATION BEFORE USE

Installation position

Surrounding safety distance for massage chair:

The horizontal interval between the backrest ascending to the highest point and descending to the lowest point should be over 50 cm;

The horizontal interval between the legrest ascending to the highest point and descending to the lowest point should be over 15 cm.

Floor protection

The floor may be worn by the massage chair, so it is advised to put some cushion material (for instance: a carpet) under the massage chair to protect the floor and reduce noises.

Floor

Carpet

Surrounding environment

Please keep the massage chair away from damp places (for example: bathroom) to avoid electric leakage.

Please keep the massage chair away from high temperature sources (for example: stove) to avoid fire or damage to leather material.

Earthing

This massage chair belongs to Class Ⅰ electrical appliance, so please use three-core power supply plug and guarantee the ground wire connected to the power supply socket has obtained good ground connection to avoid electric leakage, electric shock and some other negative effect during use.

Move method

Adjust the backrest to the highest point, place the controller and the plug of power cord on the seat cushion, be sure all the wires are not close to the floor, then press the top of the backrest with both hands to make the back two plastic foot cushions far away from the floor until the massage chair can be moved with the castors.

10th page:

USE METHOD

Turn on the power

Insert the plug into fine grounding three-pin power socket.

Turn on the power switch (installed at back lower place of backrest).

Reset

When power on, the massage chair resets to self-inspection mode;

If the self-inspection is normal, it will go to standby mode. Now the parameters can be set.

If the self-inspection is abnormal, there may be the following three display：

Err0     Internal communication is abnormal.

Err1     Massage mechanism resets abnormally.

Err2     Coin acceptor resets abnormally.

Note: refer to “Troubleshooting” part for details.

Insert coin

The coins can be inserted only when the massage chair resets to standby state. If the quantity of the inserted coin is less than the starting coin quantity, the display screen will show the current coin quantity; when the inserted coin quantity is equal to the starting coin quantity, the display screen starts to show the corresponding running time, at this moment, if no more coins are inserted within 5 seconds, the massage chair will begin to work, and the running time is the current displayed time, and the display screen will show the remaining running time. When the inserted coin quantity is more than the starting coin quantity, every inserted coin will make the running time increased accordingly (the added time:  starting time/starting coin quantity). For example: if the starting coin

quantity is 3, and the starting time is 2 (2 minutes i.e. 120 seconds), which means user just need to insert 3 coins to start the massage chair, every one coin added makes the running time increased by: 120 seconds /3=40 seconds.

Halt and reset

When the remaining time on the display screen is 0, all the massage function will be shut off, then the machine shows “rESt”, indicating the massage chairs is resetting. When the massage mechanism resetting finished, the massage chair will return to standby state, and the display screen shows“0”.

Take out the coin

- Open the front cover of coin box.

- Insert the key into the lock and take out the coin box.

Inquiry

Put the magnetic pen on the middle of the display screen, it will display the accumulated amount; the screen will return to normal state after 3 seconds when magnetic pen is removed away.

11th page:

INSTALLATION METHOD

Backrest installing/folding method

Hold up the backrest by 115° in the direction of arrow mark; the backrest is locked when a reminding sound “Ka” is heard.

Locking lever

Unfasten the locking levers situated under backrest and on the seat, then fold backrest forward in the direction of arrow mark until it touches the seat cushion.

Back cushion disassembling method

Zipper

Back cushion

Backrest

Seat cushion disassembling method

Airbag tube

Plug of airbag tube

Seat cushion

12th page:

TROUBLE SHOOTING

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| The massage chair can not be started up. | The power plug is detached. | Insert the power plug into the power socket again. |
|  | The power switch at the backside of armrest is cut off. | Switch on the power. |
|  | The fuse is burned out. | Replace a new one with the same mode. |
| Stop working suddenly | The backrest collides with wall or other obstacle. | Turn off the power switch at the backside of armrest, and move the massage chair to a spacious place, and then turn on the power again. |
|  | The massage chair undertakes overpressure (For safety, the massage mechanism will stop working once it bears overburden.) | Leave the seat temporarily, and cut off the power switch at the backside of armrest; then turn on the switch again and press the function button. |
| Coin can not be accepted. | The comparison coin is not installed correctly. | Installed the comparison coin correctly. |
| The display screen shows Err1. | The massage mechanism can not reset. | Check the massage chair; try it again after power on. If it still can not reset, please sent the massage chair to the maintenance center for repair. |
| The display screen shows Err2. | The current is instable. | Turn off the power, and start the massage chair 5 seconds later. |
| The display screen shows Err3. | The password is wrong. | Turn off the power, and start the massage chair 5 seconds later. |

Kind reminding

During use, if the massage chair makes the following sounds or you feel the following sounds, it may be the normal phenomena, and it will not influence the lifetime of massage chair.

The clack sound is produced when the massage mechanism moving up and down.

The gear or motor sounds in operation.

The friction sound is produced when the massage unit rubs against the cloth material.

Noise comes from airbag inflating and deflating.

13th page:

PRODUCT STORAGE AND MAINTENANCE

Maintenance method

Main unit

To clean the plastic, pipe and armrest with cotton fabric dipped with neutral detergent, then use dry cloth to clean them carefully.

Caution

Do not use the corrosive fluid like alcohol or cleaning powder to clean the massage chair, otherwise, the color of upholstery will fade or the massage chair will crack or be eroded.

Controller

Use the dry cloth to wipe off the dirty on the controller.

Caution

Do not clean the controller with wet towel to avoid malfunction.

Back cushion and pillow

Clean the back cushion and pillow firstly with a piece of cloth that has been dipped with neutral detergent, then use wet cloth to clean them, and finally put them in a well ventilated area.

Caution

Do not place the massage chair under direct sunlight for long time, or near the high temperature sources, such as a stove, to avoid color change or deterioration.

Storage method

Please put the massage chair in a dry place after cleaning the dust, and cover it if it is not to be used for a long period of time.

Check

During daily use, please be sure to check

Whether the massage chair has following symptoms:

a smell of burning;

When wire is touched, power is on, but sometimes not;

Power cord is overheating;

Other abnormal conditions；

Please stop use it

To prevent malfunction and accidents, please pull out the power plug from the socket and contact the dealer from whom you purchased the product or our company. Please do not disassemble it by yourself.

TECHNICAL SPECIFICATIONS

Product name: Coin-operated Massage Chair

Model: RT-M01

Rated voltage: 220v~

Rated frequency: 50 Hz

Rated power: 190W

Safety design: Class Ⅰ